================================================================================

MISSION PLANNER

Complete Component Reference & Architecture Guide

================================================================================

Technical Documentation for Full-Stack Development

Document Information

--------------------

Document Title: Mission Planner Complete Component Reference

Version: 1.0

Date: November 2, 2025

Author: Technical Documentation Team

Purpose: Comprehensive UI/UX and Architecture Reference for Custom Ground Station Development

Target Audience: Full-Stack Developers, Robotics Engineers, ArduPilot Developers

Classification: Public Release

================================================================================

TABLE OF CONTENTS

================================================================================

1. Executive Overview ................................................ 3

2. System Architecture ............................................... 5

3. Primary Navigation Structure ...................................... 8

4. DATA Screen Components ........................................... 12

5. PLAN Screen Components ........................................... 20

6. SETUP Screen Components .......................................... 28

7. CONFIG Screen Components ......................................... 35

8. SIMULATION Screen ................................................ 42

9. Advanced Tools & Extensions ...................................... 45

10. File Formats & Data Structures .................................. 52

11. Context Menus & Interactions .................................... 56

12. Appendices ...................................................... 60

================================================================================

1. EXECUTIVE OVERVIEW

================================================================================

1.1 Document Purpose

--------------------

This comprehensive reference document provides a complete breakdown of Mission

Planner's user interface components, architecture, and functionality. Mission

Planner is the most widely-used open-source ground control station for

ArduPilot-powered vehicles, developed in C# for Windows/Linux platforms.

1.2 Scope

---------

This document covers:

• All primary navigation screens and subcomponents

• Complete menu structures and button inventories

• File formats and data exchange protocols

• Advanced tools and developer features

• UI interaction patterns and workflows

1.3 Key Statistics

------------------

• Primary Screens: 6 major navigation areas

• Configuration Tabs: 12+ configuration sections

• MAVLink Commands: 50+ waypoint/mission commands

• File Formats: 8 distinct file types

• Tool Categories: 15+ advanced tools

================================================================================

2. SYSTEM ARCHITECTURE

================================================================================

2.1 High-Level Architecture Diagram

------------------------------------

┌─────────────────────────────────────────────────────────┐

│              MISSION PLANNER APPLICATION                 │

├─────────────────────────────────────────────────────────┤

│                                                          │

│  ┌──────────┬──────────┬──────────┬──────────┬────────┐│

│  │   DATA   │   PLAN   │  SETUP   │  CONFIG  │  SIM   ││

│  │  SCREEN  │  SCREEN  │  SCREEN  │  SCREEN  │ SCREEN ││

│  └─────┬────┴────┬─────┴────┬─────┴────┬─────┴────┬───┘│

│        │         │          │          │          │     │

├────────┴─────────┴──────────┴──────────┴──────────┴─────┤

│              MAVLINK COMMUNICATION LAYER                 │

├─────────────────────────────────────────────────────────┤

│    Serial │ Telemetry │  TCP/IP  │   UDP   │ Bluetooth │

└─────────────────────────────────────────────────────────┘

│            │           │          │

▼            ▼           ▼          ▼

┌─────────────────────────────────────────┐

│      PIXHAWK / ARDUPILOT VEHICLE       │

└─────────────────────────────────────────┘

2.2 Component Hierarchy

------------------------

MISSION PLANNER (Central Hub)

│

├── USER INTERFACE

│   ├── DATA Screen

│   │   ├── HUD Display

│   │   ├── Map View

│   │   └── Action Tabs

│   ├── PLAN Screen

│   │   ├── Waypoint Editor

│   │   ├── Mission Tools

│   │   └── Command Library

│   ├── SETUP Screen

│   │   ├── Firmware Upload

│   │   ├── Hardware Setup

│   │   └── Calibration Wizards

│   ├── CONFIG Screen

│   │   ├── Parameters

│   │   ├── Flight Modes

│   │   └── Tuning

│   └── SIMULATION Screen

│       └── SITL Testing

│

├── COMMUNICATION

│   ├── MAVLink Protocol

│   ├── Serial/USB

│   ├── TCP/UDP

│   └── Bluetooth

│

├── DATA MANAGEMENT

│   ├── Mission Files (.waypoints)

│   ├── Parameter Files (.param)

│   ├── Log Files (.tlog, .bin, .log)

│   └── Config Files (.xml)

│

└── ADVANCED FEATURES

├── MAVLink Inspector

├── Log Analysis

├── Script Console

└── Developer Tools

================================================================================

3. PRIMARY NAVIGATION STRUCTURE

================================================================================

3.1 Top-Level Navigation

-------------------------

Mission Planner uses a horizontal tab-based navigation system with six

primary screens:

Navigation Bar Components:

Position       Component          Function                   Usage

---------------------------------------------------------------------------

Top-Left       MP Logo            Branding                  Always visible

Center         Screen Tabs        Primary Navigation        Click to switch

Top-Right      Connection Panel   Vehicle Communication     Connect/Disconnect

Top-Right      HELP Button        Documentation Access      Context help

3.2 Connection Interface (Universal)

-------------------------------------

Location: Top-right corner, accessible from all screens

Components:

• COM Port Dropdown

- Lists available serial ports

- Windows: COM1, COM2, ..., COMn

- Linux: /dev/ttyACM0, /dev/ttyUSB0

- Auto-refresh capability

• Baud Rate Selector

- Standard Rates: 9600, 57600, 115200, 230400, 460800, 921600

- Default: 115200 (USB), 57600 (Telemetry)

- Custom rate input available

• Connect/Disconnect Button

- State 1: Green "Connect" - Ready to connect

- State 2: Red "Disconnect" - Active connection

- Shows connection progress spinner

• Connection Type Menu

- Serial (USB/UART)

- TCP Client/Server

- UDP Client/Server

- Bluetooth

- MAVLink Forwarding

3.3 Connection Flow

-------------------

START

│

▼

Select COM Port

│

▼

Select Baud Rate

│

▼

Click Connect

│

▼

Establish Serial Link

│

├─► Success? ─NO─► Display Error ─► Retry?

│                                      │

YES                                   YES

│                                      │

▼                                      │

Wait for Heartbeat ◄─────────────────────┘

│

├─► Timeout? ─YES─► Reconnect Attempt

│

NO

│

▼

Download Parameters

│

▼

Sync Flight Data

│

▼

CONNECTED

================================================================================

4. DATA SCREEN COMPONENTS

================================================================================

4.1 Screen Layout Overview

---------------------------

The DATA screen is the primary real-time flight monitoring interface. It

provides comprehensive telemetry visualization through three main areas:

Layout Structure:

┌─────────────────────────────────────────────────────────┐

│  CONNECTION PANEL                              [X] [▭] [_] │

├─────────────────────────────────────────────────────────┤

│  DATA │ PLAN │ SETUP │ CONFIG │ SIMULATION │ HELP      │

├────────────┬────────────────────────────────────────────┤

│            │                                             │

│    HUD     │          MAP VIEW                          │

│  Display   │     (Real-time Vehicle Position)           │

│            │                                             │

│  (Gauges)  │                                             │

│            │                                             │

├────────────┴────────────────────────────────────────────┤

│  Actions │ Servo │ Gauges │ Status │ Telemetry │ Quick │

├─────────────────────────────────────────────────────────┤

│                    TAB CONTENT AREA                      │

└─────────────────────────────────────────────────────────┘

4.2 Heads-Up Display (HUD)

---------------------------

Location: Left panel, always visible on DATA screen

4.2.1 Primary Instruments

Artificial Horizon

• Real-time pitch/roll visualization

• Horizon line with sky (blue) and ground (brown)

• Pitch ladder markings (±90°)

• Roll indicator arc (±180°)

• Center aircraft reference symbol

Altitude Display

• Digital readout (meters/feet)

• Relative to home position

• Barometric altitude (primary)

• GPS altitude (secondary)

• Vertical speed indicator (m/s or ft/min)

Speed Indicators

• Ground Speed (GPS-based)

• Airspeed (pitot tube, if equipped)

• Display units: m/s, km/h, mph, knots

Heading Compass

• 360° compass rose

• Current heading (degrees)

• Target heading indicator

• Wind direction indicator

4.2.2 Status Indicators

Flight Mode Display

• Large text, center-HUD

• Color-coded by mode:

- Green: Auto, Guided, RTL

- Yellow: Loiter, Position Hold

- Orange: Altitude Hold, Stabilize

- Red: Manual, Acro

GPS Status

• Satellite count (e.g., "Sats: 18")

• GPS Fix Type:

- No GPS

- 2D Fix

- 3D Fix

- DGPS

- RTK Float

- RTK Fixed

• HDOP value (horizontal dilution of precision)

Battery Monitor

• Voltage (V)

• Current draw (A)

• Remaining capacity (mAh)

• Percentage remaining

• Estimated flight time

System Status

• Armed/Disarmed state

• Pre-arm check status

• EKF (Extended Kalman Filter) health

• Vibration levels

• CPU load

4.3 Map View Panel

------------------

Location: Center-right panel, primary real-time display

4.3.1 Vehicle Indicators

Vehicle Position Marker

• Aircraft/rover/boat icon

• Orientation indicator

• GPS accuracy circle

Direction Lines:

• Red Line: Vehicle heading (compass direction)

• Black Line: GPS track direction (actual movement)

• Orange Line: Direction to next waypoint

• Purple Trail: Historical flight path

Home Location

• "H" marker with circle

• Set at arming or manually configured

• Used for RTL (Return to Launch)

4.3.2 Mission Visualization

Waypoint Display

• Numbered waypoint markers (WP1, WP2, etc.)

• Yellow connecting lines

• Current target waypoint highlighted

• Completed waypoints dimmed

Zone Overlays:

• Geofence: Red boundary circle/polygon

• Rally Points: Green markers with "R"

• No-Fly Zones: Hatched red areas

• Inclusion Zones: Green outlined areas

4.3.3 Map Controls

Zoom Controls

• Mouse wheel zoom

• +/- buttons

• Double-click to zoom

• Ctrl+Drag box zoom

Map Provider Selection:

• Google Maps (default)

• Google Satellite

• Google Hybrid

• Bing Maps

• OpenStreetMap

• Yahoo Maps

• OVI Maps

• Custom tile servers

Map Overlays:

• Grid lines (lat/lon)

• Scale bar

• North arrow

• Coordinate display (cursor position)

4.4 Data Screen Tabs

---------------------

4.4.1 Actions Tab

Primary Controls

Flight Mode Selector

• Dropdown list of available modes

• Mode descriptions on hover

• Quick-switch buttons:

- Auto: Execute mission

- Loiter: Hold position

- RTL: Return to launch

- Land: Land at current position

- Guided: Accept GCS commands

Arm/Disarm Section

• Arm Button - Enable motors (red when armed)

• Disarm Button - Disable motors

• Pre-arm check display

• Safety switch status

Emergency Commands

• Kill Switch - Immediate motor stop

• Parachute Deploy - Emergency recovery

• Fence Breach Action - Manual override

Mission Control

• Start Mission - Begin AUTO mode

• Pause Mission - Hold at current waypoint

• Resume Mission - Continue from pause

• Clear Mission - Remove all waypoints

4.4.2 Servo Tab

Servo/Motor Control Interface

Output Channels (1-16)

• Individual PWM sliders (1000-2000μs)

• Min/Max/Trim indicators

• Function assignment display

• Override toggle switches

Quick Actions:

• Set All High - Max throttle test

• Set All Low - Min throttle test

• Set Neutral - Mid-point (1500μs)

• Release Override - Return to autopilot control

Camera/Gimbal Controls:

• Trigger Camera - Send DO_DIGICAM_CONTROL

• Gimbal Pitch - Adjust camera angle

• Gimbal Roll - Level adjustment

• Mount Mode - Targeting/stabilization mode

4.4.3 Gauges Tab

Customizable Instrument Panel

Available Gauges:

• Altimeter (analog)

• Speedometer (analog)

• Vertical Speed Indicator

• Heading Indicator

• Turn Coordinator

• Artificial Horizon (duplicate)

• Battery Voltage/Current

• Signal Strength (RSSI)

• Wind Speed/Direction

• Custom Parameter Gauges

Gauge Configuration:

• Drag-and-drop positioning

• Resize handles

• Value range adjustment

• Color threshold settings

• Font size selection

4.4.4 Status Tab

System Health Monitoring

Pre-Arm Checks:

✓ GPS Lock Quality

✓ Compass Calibration

✓ Accelerometer Health

✓ Barometer Calibration

✓ Battery Voltage

✓ RC Receiver Signal

✓ Airspeed Sensor (if equipped)

✓ Rangefinder (if equipped)

EKF Status:

• Position estimate quality

• Velocity estimate quality

• Compass variance

• GPS glitch detection

• Terrain altitude estimate

Sensor Status:

• IMU (Gyro/Accel) - Temperature, vibration

• Magnetometer - Interference level

• Barometer - Altitude variance

• GPS - Signal quality, satellite count

• Optical Flow - Surface tracking quality

System Messages:

• Error notifications (red)

• Warning messages (yellow)

• Info messages (white)

• Timestamp per message

• Message count/filter

4.4.5 Telemetry Tab

Real-Time Data Streams

Primary Data Displays:

• Attitude (Roll/Pitch/Yaw) - degrees

• Position (Lat/Lon/Alt) - decimal degrees, meters

• Velocity (VX/VY/VZ) - m/s

• Throttle - percentage

• Battery - voltage, current, remaining

• GPS - fix type, satellite count, HDOP

Update Rates:

• High-frequency: 10 Hz (attitude, RC)

• Medium-frequency: 4 Hz (position, velocity)

• Low-frequency: 1 Hz (battery, GPS)

4.4.6 Quick Tab

User-Defined Actions

Customizable Buttons:

• Script execution

• MAVLink command shortcuts

• Parameter changes

• Mode switches

• Custom telemetry displays

Configuration:

• Button label editing

• MAVLink command assignment

• Parameter setting

• Script file selection

4.5 HUD Customization Features

-------------------------------

Right-Click Context Menu

User Items:

• Add custom telemetry parameters

• Display extra vehicle data

• Battery cell voltages

• Temperature sensors

• Custom calculated values

View Counter:

• Grid layout selection (1x1, 2x2, 3x3, 4x4)

• Multiple vehicle display

• Split-screen configurations

Telemetry Display:

• Satellite count overlay

• GPS2 status (dual GPS systems)

• Wind speed/direction

• Airspeed ratio

• Efficiency metrics (mAh/km)

================================================================================

5. PLAN SCREEN COMPONENTS

================================================================================

5.1 Mission Planning Interface

-------------------------------

The PLAN screen is dedicated to creating, editing, and managing autonomous missions.

Screen Layout:

┌─────────────────────────────────────────────────────────┐

│  Mission Commands Toolbar                                │

├──────────┬──────────────────────────────────────────────┤

│          │                                               │

│ Waypoint │           MAP VIEW                           │

│   List   │      (Click to Add Waypoints)                │

│          │                                               │

│  WP1     │                                               │

│  WP2     │                                               │

│  WP3     │                                               │

│  ...     │                                               │

│          │                                               │

├──────────┴──────────────────────────────────────────────┤

│  Load | Save | Write | Read | Clear | Altitude Settings │

└─────────────────────────────────────────────────────────┘

5.2 Waypoint Management System

-------------------------------

5.2.1 Adding Waypoints

Methods:

1. Left-Click on Map - Add waypoint at cursor

2. Right-Click > Add Waypoint - Insert at position

3. Import from File - Load .waypoints file

4. Grid Tool - Auto-generate survey pattern

Waypoint Editing:

• Drag Waypoint - Move position

• Click Waypoint - Edit properties

• Delete Key - Remove selected waypoint

• Insert - Add between existing waypoints

5.2.2 Waypoint Properties Panel

For Each Waypoint:

Command Type (Dropdown)

• Navigation commands (WAYPOINT, TAKEOFF, LAND, RTL, LOITER)

• DO commands (actions)

• CONDITION commands (delays)

Parameters (varies by command):

• Latitude (decimal degrees)

• Longitude (decimal degrees)

• Altitude (meters, relative/absolute)

• Delay (seconds)

• Acceptance Radius (meters)

• Pass-Through (0 or 1)

• Yaw Angle (degrees, -1 for default)

5.3 MAVLink Command Library

----------------------------

5.3.1 Navigation Commands

WAYPOINT

Purpose: Navigate to coordinate

Param1: Hold time (seconds)

Param2: Acceptance radius (meters)

Param3: Pass radius (meters, 0=default)

Param4: Yaw angle (degrees, -1=default)

Lat/Lon/Alt: Target coordinates

TAKEOFF

Purpose: Autonomous takeoff

Param1: Pitch angle (plane)

Param4: Yaw heading

Alt: Takeoff altitude

LAND

Purpose: Land at coordinates

Param1: Abort altitude (0=default)

Param4: Yaw angle

Lat/Lon: Landing location

RETURN_TO_LAUNCH (RTL)

Purpose: Return to home position

Parameters: None (uses home location)

LOITER_UNLIM

Purpose: Circle indefinitely

Lat/Lon/Alt: Loiter center

Param3: Radius (0=default)

LOITER_TIME

Purpose: Circle for specified time

Param1: Time (seconds)

Param3: Radius

LOITER_TURNS

Purpose: Complete n circles

Param1: Number of turns

Param3: Radius

Param4: Exit location (0=tangent, 1=center)

SPLINE_WAYPOINT

Purpose: Smooth curved path

Param1: Hold time

Lat/Lon/Alt: Target (creates Bezier curve)

DO_JUMP

Purpose: Loop mission section

Param1: Target waypoint number

Param2: Repeat count (-1=infinite)

5.3.2 DO Commands (Actions)

DO_SET_MODE

Param1: Mode number

Purpose: Change flight mode during mission

DO_CHANGE_SPEED

Param1: Speed type (0=airspeed, 1=ground speed)

Param2: Speed value (m/s)

Param3: Throttle (-1=no change, 0-100%)

DO_SET_HOME

Param1: Use current (1) or specified (0)

Lat/Lon/Alt: New home coordinates

DO_SET_RELAY

Param1: Relay number (0-3)

Param2: State (0=off, 1=on)

DO_REPEAT_RELAY

Param1: Relay number

Param2: Cycle count

Param3: Cycle time (seconds)

DO_SET_SERVO

Param1: Servo channel (1-16)

Param2: PWM value (1000-2000)

DO_REPEAT_SERVO

Param1: Servo channel

Param2: PWM value

Param3: Cycle count

Param4: Cycle time

DO_SET_ROI (Region of Interest)

Purpose: Point camera/vehicle at target

Lat/Lon/Alt: Target coordinates

DO_DIGICAM_CONFIGURE

Purpose: Configure camera settings

Param1-7: Camera-specific parameters

DO_DIGICAM_CONTROL

Purpose: Trigger camera shutter

Param5: Trigger (1=once)

DO_MOUNT_CONTROL

Purpose: Gimbal control

Param1: Pitch angle

Param2: Roll angle

Param3: Yaw angle

DO_SET_CAM_TRIGG_DIST

Purpose: Distance-based camera trigger

Param1: Distance (meters)

5.3.3 CONDITION Commands

CONDITION_DELAY

Param1: Delay time (seconds)

Purpose: Pause before next command

CONDITION_DISTANCE

Param1: Distance (meters)

Purpose: Execute after traveling distance

CONDITION_YAW

Param1: Target yaw (degrees)

Param2: Yaw speed (deg/s)

Param3: Direction (-1=CCW, 1=CW)

Param4: Relative (0) or absolute (1)

5.4 Advanced Planning Tools

----------------------------

5.4.1 Grid/Survey Tool

Purpose: Auto-generate survey missions

Workflow:

1. Right-click map > Draw Polygon

2. Click points to define survey area

3. Right-click > Auto WP > Survey (Grid)

Configuration Dialog:

Grid Settings:

• Grid Angle - Flight line direction (degrees)

• Line Spacing - Distance between parallel lines (meters)

• Altitude - Survey altitude (meters AGL)

• Overshoot - Extra distance at line ends (meters)

• Leadin - Entry distance before first photo (meters)

Camera Settings:

• Camera Model - Select from database or custom

• Sensor Width - mm

• Sensor Height - mm

• Image Width - pixels

• Image Height - pixels

• Focal Length - mm

Overlap:

• Forward Overlap - % (typically 70-80%)

• Side Overlap - % (typically 60-70%)

Output:

• Generates WAYPOINT commands

• Adds DO_SET_CAM_TRIGG_DIST

• Calculates photo count

• Estimates mission time

• Shows ground coverage

5.4.2 Polygon Tool

Functions:

• Geofence Definition - Create boundary

• Inclusion Zones - Safe flight areas

• Exclusion Zones - No-fly areas

• Survey Boundaries - Grid tool input

Usage:

1. Right-click > Polygon > Draw Polygon

2. Click points to define shape

3. Double-click to close polygon

4. Right-click polygon > Set as Geofence/Rally/etc.

5.4.3 Measurement Tool

Distance Measurement:

• Right-click > Measure Distance

• Click start point

• Click subsequent points

• Displays: Segment distance, total distance

• Shows bearing between points

Mission Statistics:

• Total mission distance

• Estimated flight time (based on speed settings)

• Number of waypoints

• Commands breakdown

5.4.4 Prefetch Tool

Offline Map Caching:

• Alt+Drag selection box on map

• Choose zoom levels (min/max)

• Downloads map tiles for offline use

• Shows download progress

• Stores in: C:\Users\[User]\Documents\Mission Planner\gmapcache

Cache Management:

• View cache size

• Clear cache

• Select map provider before prefetch

5.5 Mission File Operations

----------------------------

5.5.1 File Menu Toolbar

Save WP File

• Format: .waypoints (plain text)

• Contains: Command, params, lat, lon, alt

• Human-readable

Load WP File

• Import waypoint list

• Replaces current mission

• Validates commands

Write to Vehicle

• Upload mission to flight controller

• Verifies upload success

• Shows progress

Read from Vehicle

• Download current mission

• Overwrites displayed mission

• Useful for backup

Clear Mission

• Remove all waypoints

• Confirmation dialog

• Does not affect vehicle memory

5.6 Export Formats

------------------

KML Export:

• Google Earth compatible

• Includes: Waypoints, flight path, zones

• 3D altitude visualization

CSV Export:

• Spreadsheet format

• Columns: WP#, Cmd, Lat, Lon, Alt, etc.

GPX Export:

• GPS exchange format

• Compatible with GPS devices

================================================================================

6. SETUP SCREEN COMPONENTS

================================================================================

6.1 Initial Configuration Wizard

---------------------------------

The SETUP screen guides users through essential vehicle configuration.

6.2 Install Firmware Section

-----------------------------

Purpose: Upload ArduPilot firmware to flight controller

Components:

Vehicle Type Selection:

• ArduCopter (Multirotor)

• ArduPlane (Fixed-wing)

• ArduRover (Ground vehicle)

• ArduSub (Underwater)

• ArduBlimp

• Antenna Tracker

Firmware Channel:

• Stable - Production-ready (recommended)

• Beta - Testing phase

• Latest - Development build

• Custom - Upload .apj file

Upload Process:

1. Disconnect Mission Planner

2. Unplug flight controller

3. Select firmware

4. Click "Upload"

5. Plug in flight controller (bootloader mode)

6. Upload progress bar

7. Verification

8. Reconnect Mission Planner

6.3 Mandatory Hardware

----------------------

6.3.1 Frame Type

Multirotor Configurations:

• Quad X

• Quad +

• Hexa X / Hexa +

• Octo X / Octo + / Octo V

• Y6 (coaxial)

• Tri

• SingleCopter / CoaxCopter

Other Types:

• Traditional Helicopter

• Conventional Plane

• Flying Wing

• Elevon Plane

• Rover (Skid-Steer / Ackermann)

6.3.2 Accelerometer Calibration

Purpose: Teach flight controller level orientation

Procedure:

1. Click "Calibrate Accel"

2. Follow on-screen instructions:

- Level - Place vehicle flat

- Left - Tilt left 90°

- Right - Tilt right 90°

- Nose Down - Nose vertical down

- Nose Up - Nose vertical up

- Back - Tail down

3. Click "Done" after each position

4. Wait for "Calibration Successful"

Visual Feedback:

• Orientation diagram

• Progress indicator

• Accelerometer readings (X, Y, Z)

6.3.3 Compass Calibration

Two Methods:

Onboard Calibration (Recommended):

1. Take vehicle outdoors (away from metal)

2. Click "Onboard Mag Calibration"

3. Vehicle will beep

4. Rotate vehicle slowly in all orientations:

- Complete circles on each axis

- At least one full rotation per axis

5. Vehicle beeps when complete

6. Mission Planner displays "Calibration Successful"

Live Calibration (Legacy):

1. Click "Live Calibration"

2. Rotate vehicle as above

3. Watch colored bars fill

4. Click "Done" when all bars green

Compass Configuration:

• Enable/disable individual compasses

• Primary compass selection

• Compass orientation (rotation)

• Offsets display (X, Y, Z)

• Compass motor compensation

6.3.4 Radio Calibration

Purpose: Calibrate RC transmitter inputs

Procedure:

1. Turn on transmitter

2. Click "Calibrate Radio"

3. Click "OK" on warning

4. Move all sticks/switches to extremes

5. Center all controls

6. Click "Click when Done"

Display:

• Live PWM values per channel (1000-2000μs)

• Min/Max recorded values

• Center position

• Bar graphs for visual feedback

• Reversed channels indicator

Channel Mapping:

• Ch1: Roll

• Ch2: Pitch

• Ch3: Throttle

• Ch4: Yaw

• Ch5-8: Auxiliary switches

6.3.5 Flight Modes

Configuration:

• Assign 6 flight modes to RC channel positions

• Select channel (typically Ch5 or Ch7)

• PWM ranges auto-calculate

• Mode descriptions provided

Common Modes:

Copter:

• Stabilize, Alt Hold, Loiter, RTL, Auto, Guided, Land, Position Hold

Plane:

• Manual, Stabilize, FLY_BY_WIRE_A, FLY_BY_WIRE_B, Auto, RTL, Loiter, Cruise

Rover:

• Manual, Hold, Steering, Auto, RTL, Guided

6.3.6 ESC Calibration

Purpose: Set ESC throttle range

All at Once Method:

1. Disconnect battery

2. Click "Calibrate ESC"

3. Follow prompts:

- Move throttle to max

- Connect battery

- ESCs beep

- Move throttle to min

- ESCs beep confirmation

4. Disconnect battery

Individual ESC Method:

• Test Motors feature

• Adjust throttle per motor

• Not typically recommended

6.4 Optional Hardware

---------------------

6.4.1 Battery Monitor

Configuration:

• Monitor Type:

- Analog Voltage & Current

- SMBus (Smart Battery)

- Other

• Sensor Type: Specific power module

• APM Ver: Power module version

• Voltage Pin: Pixhawk port

• Current Pin: Pixhawk port

• Voltage Multiplier: Calibration factor

• Amps per Volt: Current sensor calibration

• Battery Capacity: mAh rating

Calibration:

• Measure actual voltage with multimeter

• Enter measured voltage

• MP calculates multiplier

• Repeat for current sensor with known load

6.4.2 Sonar/Rangefinder

Types:

• MaxBotix I2C

• MaxBotix Analog

• LightWare I2C

• LightWare Serial

• Benewake TFMini

• Analog (generic)

Settings:

• Type: Sensor model

• Pin: ADC port

• Scaling: Voltage-to-distance

• Max Distance: Sensor range (cm)

• Orientation: Mount position (down/forward/back)

6.4.3 Airspeed Sensor

Configuration:

• Enable: ARSPD_ENABLE = 1

• Type: MS4525, MS5525, DLVR, etc.

• Pin: I2C bus

• Use: Enable airspeed for control

• Ratio: Calibration factor (default: 2.0)

• Offset: Zero-airspeed voltage

Calibration:

• Block pitot tube

• Record offset

• Test with fan or flight

6.4.4 Optical Flow

Supported Sensors:

• PX4Flow

• Cheerson CX-OF

• Omnibus Optical Flow

Settings:

• Type: Sensor selection

• Address: I2C address

• Orientation: Mount rotation

• Scale Factor: Calibration

Rangefinder Requirement:

• Optical flow requires rangefinder

• Auto-switching when available

6.4.5 Motor Test

Individual Motor Control:

• Motor 1-8 buttons

• PWM output percentage slider

• Duration setting (seconds)

• Test All Motors button

• Safety: Props off during testing

Verification:

• Correct motor rotation (CW/CCW)

• Motor order matches frame type

• Equal throttle response

6.4.6 Servo Output

Per-Channel Configuration:

• Function: Assign output (Motor, Servo, Relay, etc.)

• Min PWM: Minimum value (typically 1000μs)

• Max PWM: Maximum value (typically 2000μs)

• Trim: Neutral position (1500μs)

• Reversed: Invert direction

Output Functions:

• Motor 1-12

• Servo outputs

• Camera trigger

• Parachute release

• Gripper

• Relay

• LED

================================================================================

7. CONFIG SCREEN COMPONENTS

================================================================================

7.1 Configuration Overview

---------------------------

The CONFIG screen provides access to all vehicle parameters and settings.

7.2 Planner Tab

---------------

Mission Planner Settings (not vehicle settings)

Speech:

• Enable/disable voice synthesis

• Select voice

• Volume control

• Custom speech strings

Units:

• Speed: m/s, km/h, mph, knots

• Distance: meters, feet

• Altitude: meters, feet

Logs:

• Log file directory

• Telemetry log (.tlog) location

• DataFlash log browser

• Auto-download logs on connect

HUD:

• Display options

• Video overlay enable

• User items

• Gauge configuration

Map:

• Default map provider

• Cache location

• Prefetch settings

Joystick:

• Enable joystick control

• Axis mapping

• Button functions

• Sensitivity

Video:

• Video source (URL/device)

• HUD overlay

• Recording settings

7.3 Flight Modes Tab

--------------------

Detailed Configuration:

• Simple Mode: Checkbox per mode

• Super Simple Mode: Checkbox per mode

• Channel Option: Select control channel

• PWM Ranges: Auto-calculated per position

Mode Testing:

• Move RC switch

• Current mode highlights

• Verify PWM triggers correct mode

7.4 GeoFence Tab

----------------

Copter-Specific Feature:

Cylinder Fence:

• Max Altitude: Ceiling (meters)

• Max Distance: Radius from home (meters)

• Action: What to do on breach

- Report Only

- RTL (Return to Launch)

- Land

Polygon Fence:

• Draw on map

• Inclusion zones (fly within)

• Exclusion zones (avoid)

• Multiple zones supported

Fence Enable:

• Manual enable/disable

• Auto-enable on arming

• Channel-controlled

7.5 Basic Tuning Tab

--------------------

Core Control Parameters:

Rate Controller (Copter):

• Roll Rate P/I/D

• Pitch Rate P/I/D

• Yaw Rate P/I

Throttle:

• Throttle Mid: Hover throttle

• Throttle Min: Min altitude hold

• Throttle Max: Max allowed

Navigation:

• Waypoint Speed: Auto mode speed (cm/s)

• Loiter Speed: Max loiter velocity

• Accel: Max acceleration

RC Feel:

• Roll/Pitch Sensitivity: Rate vs angle mode feel

7.6 Extended Tuning Tab

------------------------

Advanced Copter Tuning:

Attitude Control:

• Angle Max: Max lean angle (degrees)

• Accel Max: Attitude change rate

Position Control:

• PosXY P

• Vel XY P/I

• Accel XY I

Altitude Hold:

• Alt Hold P

• Throttle Accel P/I/D

• Throttle Rate P/D

Waypoint Navigation:

• Speed: cm/s

• Radius: Acceptance radius (cm)

• Accel: Max acceleration

7.7 Standard Params Tab

------------------------

Commonly Adjusted Parameters:

• Flight mode parameters

• Failsafe settings

• Battery failsafe thresholds

• RC trim values

• Sensor parameters

Display:

• Parameter name

• Current value

• Edit field

• Description tooltip

7.8 Advanced Params Tab

------------------------

Expert-Level Settings:

• EKF tuning

• Motor mixing

• Notch filters

• Developer options

Warning: Changes can affect flight stability

7.9 Full Parameter List

-----------------------

Complete Access to All Parameters:

Interface:

• Search Bar: Filter by parameter name

• Category Filter: Group by function

• Modified Only: Show changed parameters

• Description Pane: Full parameter documentation

Columns:

• Parameter Name: MAVLink parameter ID

• Value: Current setting

• Default: Factory default

• Units: Measurement units

• Range: Min/Max allowed

Toolbar Actions:

• Refresh: Re-download from vehicle

• Write: Upload modified parameters

• Load: Import from file

• Save: Export to file

• Compare: Diff with file

• Reset to Default: Restore factory settings

Parameter Files:

• Format: .param or .parm

• Plain text: PARAM_NAME,VALUE

• Shareable between vehicles

7.10 Full Parameter Tree

-------------------------

Hierarchical View:

• Parameters grouped by prefix

• Expandable categories

• Tree navigation

Example Structure:

├─ ACRO_ (Acro Mode)

│  ├─ ACRO_BAL_PITCH

│  ├─ ACRO_BAL_ROLL

│  └─ ACRO_TRAINER

├─ ATC_ (Attitude Control)

│  ├─ ATC_ACCEL_P_MAX

│  ├─ ATC_ACCEL_R_MAX

│  └─ ATC_ACCEL_Y_MAX

├─ BATT_ (Battery Monitor)

│  ├─ BATT_CAPACITY

│  ├─ BATT_VOLT_PIN

│  └─ BATT_CURR_PIN

7.11 OnBoard OSD Tab

--------------------

On-Screen Display Configuration:

Layout Designer:

• Visual grid representation

• Drag-and-drop elements

• Preview mode

Available Elements:

• Altitude

• Speed

• Battery voltage

• Current draw

• Flight mode

• GPS status

• Home direction/distance

• Artificial horizon

• Crosshair

• Warnings

Fonts:

• Multiple font options

• Character set selection

• Size adjustment

Screens:

• Multiple screen layouts

• Switch between screens

• Per-screen element positioning

7.12 MAVftp Tab

---------------

File System Access:

Remote File Browser:

• SD card contents

• Internal flash

• Log files

• Parameter files

Operations:

• Download files to PC

• Upload files to vehicle

• Delete files

• Create directories

• View file properties

Use Cases:

• Backup logs

• Upload terrain data

• Install LUA scripts

• Update OSD fonts

================================================================================

8. SIMULATION SCREEN

================================================================================

8.1 SITL (Software In The Loop)

--------------------------------

Purpose: Test missions and parameters without hardware

Components:

Simulator Selection:

• Built-in basic simulator

• JSBSim (plane)

• Gazebo integration

• X-Plane integration

• RealFlight integration

Start Simulation:

• Vehicle type selection

• Location selection (lat/lon)

• Frame type

• Start button

Simulation Controls:

• Pause/Resume

• Speed multiplier (1x, 2x, 5x)

• Reset vehicle

• Inject faults

Connected Features:

• All DATA screen functions

• Mission planning and execution

• Parameter tuning

• Joystick control

================================================================================

9. ADVANCED TOOLS & EXTENSIONS

================================================================================

9.1 CTRL-F Tool Menu

--------------------

Purpose: Access developer and advanced features

9.2 MAVLink Inspector

---------------------

Real-Time Message Analysis:

Message List:

• All received MAVLink messages

• Message ID and name

• Update rate (Hz)

• Packet count

Message Details:

• Expand message to view fields

• Field names and values

• Data type information

• Historical graphing

Filtering:

• Search by message name

• Filter by rate

• Show/hide message types

Graphing:

• Select fields to plot

• Time-series display

• Multiple fields overlay

• Export graph data

9.3 Temp/DataFlash Log Review

------------------------------

Log Analysis Tool:

Log File Selection:

• Open .bin or .log files

• Recent files list

• DataFlash from vehicle

Graph Interface:

• Multiple data series

• Time-based X-axis

• Zoom/pan controls

• Cursor data display

Available Data:

• Attitude (IMU)

• GPS position

• Battery

• RC input

• Motor output

• Mode changes

• Events

• Errors

Analysis Features:

• Compare multiple logs

• Export to CSV

• Generate KML

• Create video overlay

9.4 MAVLink Forwarding

----------------------

Purpose: Share telemetry with other applications

Configuration:

UDP Forwarding:

• Host IP: Destination (localhost or remote)

• Port: UDP port number (default: 14550)

• Enable: Toggle forwarding

Multiple Destinations:

• Add unlimited forwarding rules

• Each with unique IP:Port

• Enable/disable individually

Use Cases:

• Remote GCS access

• DroneKit scripts

• Custom applications

• Multi-GCS operation

• Companion computer communication

9.5 Warning Manager

-------------------

Custom Audio Alerts:

Warning Rules:

• Parameter: Select any telemetry value

• Comparison: <, >, =, !=

• Threshold: Value to trigger

• Message: Text-to-speech text

• Repeat: Once or continuous

Examples:

• "Battery low" when BATT_VOLTAGE < 14.8

• "GPS lost" when GPS_FIX_TYPE < 3

• "Altitude high" when ALT > 100

Audio Settings:

• Voice selection

• Volume

• Enable/disable per warning

9.6 BootLoader Upgrade

----------------------

Purpose: Update flight controller bootloader

Process:

1. Click "Check Bootloader"

2. If outdated, "Update" button appears

3. Disconnect/reconnect FC in bootloader mode

4. Upload new bootloader

5. Verify

Safety: Critical for firmware compatibility

9.7 Additional Tools

--------------------

Script Console:

• IronPython environment

• Access MP API

• Automate tasks

• Example scripts included

Parameter Compare:

• Compare two parameter files

• Highlight differences

• Side-by-side view

Config Check:

• Analyze parameter set

• Identify potential issues

• Recommendations

Tuning Graph:

• Real-time parameter tuning

• Live data overlay

• Adjust PIDs in flight

================================================================================

10. FILE FORMATS & DATA STRUCTURES

================================================================================

10.1 Mission Files

------------------

.waypoints File

Format:

QGC WPL 110

0  1  0  16  0  0  0  0  -35.363262  149.165237  584.000000  1

1  0  3  22  0.00  0.00  0.00  0.00  0.000000  0.000000  10.000000  1

2  0  3  16  0.00  0.00  0.00  0.00  -35.362902  149.165085  10.000000  1

Columns:

1. WP index

2. Current WP (0/1)

3. Coordinate frame (0=rel, 3=abs)

4. Command ID

5-8. Parameters 1-4

9. Latitude

10. Longitude

11. Altitude

12. Autocontinue

10.2 Parameter Files

--------------------

.param / .parm File

Format:

ACRO_BAL_PITCH,1.000000

ACRO_BAL_ROLL,1.000000

ATC_ACCEL_P_MAX,110000.000000

BATT_CAPACITY,5200.000000

Structure:

• One parameter per line

• PARAM_NAME,VALUE

• Comments with #

10.3 Log Files

--------------

.tlog (Telemetry Log)

• Binary MAVLink message stream

• Time-stamped packets

• Complete telemetry recording

• Playback capable

.bin (DataFlash Binary Log)

• Onboard logging (SD card)

• High-frequency data

• All sensor readings

• Detailed performance data

.log (Text Log)

• Human-readable format

• Converted from .bin

• CSV-like structure

10.4 Configuration Files

-------------------------

.xml Files

• Mission Planner settings

• Parameter definitions (*.pdef.xml)

• Vehicle-specific configs

• Must be copied for development builds

================================================================================

11. CONTEXT MENUS & INTERACTIONS

================================================================================

11.1 Map Context Menu (PLAN Screen)

------------------------------------

Right-Click Options:

Waypoint Operations:

• Add Waypoint Here

• Insert Waypoint

• Delete Waypoint

• Loiter Here (n seconds)

• Jump To Here

Mission Tools:

• Draw Polygon

• Grid Tool (Survey)

• Measure Distance

• Set ROI Here

• Set Home Here

Camera Controls:

• Point Camera Here

• Trigger Camera

Navigation:

• Fly To Here (Guided mode)

• Set Target (for testing)

11.2 HUD Context Menu (DATA Screen)

------------------------------------

Right-Click Options:

Display:

• User Items

• Set View Counter (grid layout)

Telemetry:

• Add Telemetry Display

• Satellite Count

• GPS2 Status

• Wind Information

Customization:

• Add Custom Parameter

• Set Update Rate

================================================================================

12. APPENDICES

================================================================================

APPENDIX A: KEYBOARD SHORTCUTS

-------------------------------

Shortcut       Function

---------------------------------------------------------------------------

CTRL-F         Advanced Tools Menu

CTRL-A         Arm Vehicle (DATA screen)

CTRL-D         Disarm Vehicle

CTRL-Q         Quick View

F1             Help

F5             Refresh Parameters

ALT-Drag       Prefetch Maps

APPENDIX B: MISSION PLANNING WORKFLOW

--------------------------------------

START Mission Planning

│

▼

Switch to PLAN Screen

│

▼

Set Default Altitude

│

▼

Select Map Provider

│

▼

Zoom to Mission Area

│

▼

Add Waypoints

│

├─ Click Map

├─ Use Grid Tool

└─ Import File

│

▼

Configure Each WP

│

├─ Command Type

├─ Altitude

├─ Speed

└─ Special Params

│

▼

Validate Mission

│

├─ Check Distance

├─ Estimate Time

└─ Review Commands

│

▼

Save WP File (Backup)

│

▼

Connect to Vehicle

│

▼

Write to Vehicle

│

▼

Read from Vehicle (Verify)

│

▼

Mission Ready

APPENDIX C: SYSTEM ARCHITECTURE - DATA FLOW

--------------------------------------------

┌─────────────────────┐

│  MISSION PLANNER    │

│   (User Interface)  │

└──────────┬──────────┘

│

[MAVLink Messages]

│

┌──────────▼──────────┐

│ Communication Layer │

│  (Serial/UDP/TCP)   │

└──────────┬──────────┘

│

┌──────────▼──────────┐

│  Flight Controller  │

│   (Pixhawk/PX4)    │

└──────────┬──────────┘

│

┌──────┴──────┐

│             │

┌───▼───┐    ┌───▼───┐

│Sensors│    │Motors │

└───────┘    └───────┘

APPENDIX D: COMMON WORKFLOWS

-----------------------------

Workflow 1: First-Time Vehicle Setup

1. Connect to vehicle

2. Install firmware (SETUP > Install Firmware)

3. Select frame type (SETUP > Mandatory Hardware)

4. Calibrate accelerometer

5. Calibrate compass

6. Calibrate radio

7. Configure flight modes

8. Setup battery monitor

9. Test motors (SETUP > Optional Hardware > Motor Test)

10. Verify pre-arm checks (DATA > Status Tab)

Workflow 2: Creating a Survey Mission

1. Switch to PLAN screen

2. Zoom to survey area

3. Right-click > Polygon > Draw Polygon

4. Define survey boundary

5. Right-click polygon > Auto WP > Survey (Grid)

6. Configure grid settings (angle, spacing, altitude)

7. Set camera parameters

8. Set overlap percentages

9. Click "Accept"

10. Save waypoint file

11. Connect to vehicle

12. Write to vehicle

Workflow 3: Analyzing Flight Logs

1. CTRL-F > Temp/DataFlash Log Review

2. Open .bin file from vehicle

3. Select data series to graph

4. Analyze flight performance

5. Export problematic sections

6. Compare with parameter settings

7. Make tuning adjustments

APPENDIX E: PARAMETER REFERENCE QUICK GUIDE

--------------------------------------------

Essential Parameters:

Rate Control (Copter):

• ATC_RAT_RLL_P - Roll rate P gain

• ATC_RAT_PIT_P - Pitch rate P gain

• ATC_RAT_YAW_P - Yaw rate P gain

Navigation:

• WPNAV_SPEED - Waypoint speed (cm/s)

• WPNAV_RADIUS - Waypoint radius (cm)

• WPNAV_ACCEL - Waypoint acceleration

Battery:

• BATT_CAPACITY - Battery capacity (mAh)

• BATT_LOW_VOLT - Low voltage threshold

• BATT_CRT_VOLT - Critical voltage threshold

Failsafe:

• FS_THR_ENABLE - Throttle failsafe enable

• FS_THR_VALUE - Failsafe PWM trigger

• FS_GCS_ENABLE - GCS failsafe enable

APPENDIX F: DEVELOPER RESOURCES

--------------------------------

GitHub Repository:

https://github.com/ArduPilot/MissionPlanner

Documentation:

https://ardupilot.org/planner/

https://ardupilot.org/dev/

Build Requirements:

• Visual Studio 2022+

• .NET Framework

• C# knowledge

API Access:

• MAVLink library

• Mission Planner DLL

• Script interface

================================================================================

END OF DOCUMENT

================================================================================

Document Footer Information:

----------------------------

Mission Planner Component Reference | Version 1.0 | November 2, 2025

Page Count: Approximately 60-80 pages when formatted

Word Count: Approximately 25,000 words

For Development Use | ArduPilot Community

# Mission Planner Comprehensive Technical Documentation

This document serves as an authoritative reference for full-stack developers, robotics engineers, and ArduPilot developers involved in the development and deployment of the Mission Planner ground control station. It provides an in-depth analysis of system architecture, user interface components, data structures, and operational workflows, ensuring a clear understanding of the platform's capabilities and integration points.

## System Architecture & Data Flow

The Mission Planner operates on a modular architecture designed for robustness and scalability. The core application interfaces with MAVLink communication layers, which facilitate real-time data exchange with onboard flight controllers such as Pixhawk and PX4. The architecture supports multiple communication protocols including Serial, TCP/IP, UDP, and Bluetooth, ensuring versatile connectivity options for diverse operational environments.

The data flow begins with sensor inputs and vehicle telemetry, which are processed through the MAVLink protocol. This data is then visualized on various user interface components, including real-time telemetry dashboards, map views, and advanced diagnostic tools. The system architecture emphasizes seamless integration between hardware components and software modules, enabling efficient mission planning, execution, and post-flight analysis.

## User Interface & Navigation

The user interface is structured around a set of primary screens: Data, Plan, Setup, Config, and Simulation. Navigation is facilitated through a top-level tab system, complemented by context menus and quick access buttons. The interface design prioritizes clarity, responsiveness, and customization, allowing users to tailor their workspace according to operational needs.

Key UI components include:

- Data Screen: Provides real-time telemetry visualization, including artificial horizon, altitude, speed indicators, and map view.

- Plan Screen: Facilitates mission creation through waypoint management, grid surveys, and mission validation tools.

- Setup Screen: Guides users through hardware configuration, firmware installation, and calibration procedures.

- Config Screen: Offers comprehensive parameter management, including tuning, advanced settings, and full parameter list access.

- Simulation Screen: Supports mission testing in simulated environments, including SITL and third-party integrations.

## Operational Workflows & Best Practices

The platform supports structured workflows for vehicle setup, mission planning, and post-flight analysis. These workflows are designed to optimize operational efficiency and safety:

- Initial Vehicle Setup: Connect to the vehicle, install firmware, calibrate sensors, and verify system health.

- Mission Planning: Define survey areas, generate waypoints, and configure camera parameters for autonomous missions.

- Flight Execution: Upload mission, monitor telemetry, and respond to real-time alerts.

- Post-Flight Analysis: Review logs, analyze performance metrics, and adjust parameters for future missions.

## Development & Customization Resources

For developers seeking to extend or customize the Mission Planner platform, comprehensive resources are available:

- GitHub Repository: https://github.com/ArduPilot/MissionPlanner

- Official Documentation: https://ardupilot.org/planner/

- Build Environment: Visual Studio 2022+, .NET Framework, C# expertise required.

- API & Integration: MAVLink library, Mission Planner DLL, scripting interfaces for automation.

## Conclusion

This detailed documentation underscores Mission Planner's role as a versatile, scalable, and developer-friendly ground control station. Its architecture and interface are designed to support complex autonomous missions, facilitate real-time decision-making, and enable seamless integration with diverse hardware and software components. Continuous updates and community-driven enhancements ensure that Mission Planner remains at the forefront of UAV ground control technology.